DATA SHEET for LED

| Part No. | LG81030 |  |  |
| --- | --- | --- | --- |
| ###### Emitted Color |  | ###### Lens Color | ###### Chip Material |
| Green |  | Color Diffused | GaP |

Absolute Maximum Rating of Each Segment (Ta = 25℃)

| Parameter | Symbol | Maximum Rating | Unit |
| --- | --- | --- | --- |
| Power Dissipation | PM | 80 | mW |
| Pulse Forward Current (Duty 1/10 @ 1kHz) | IFP | 100 | mA |
| Continuous Forward Current | IF | 30 | mA |
| Reverse Voltage | VR | 6 | V |
| Operation Temperature | Topr | －25℃∼85℃ | ℃ |
| Storage Temperature | Tstg | －40℃∼100℃ | ℃ |
| Soldering Temperature : 2.0mm from Body for 3 seconds at 260℃ |  |  |  |

Electron-Optical Characteristics of Each Segment (Ta = 25℃)

| Parameter | Symbol | Min. | Typ. | Max. | Unit | Test Condition |
| --- | --- | --- | --- | --- | --- | --- |
| Luminous Intensity | IV |  | 30 |  | mcd | IF=20 mA |
| Forward Voltage | VF |  | 2.2 | 2.5 | V | IF=20 mA |
| Reverse Current | IR |  |  | 20 | μA | VR=5V |
| Dominant Wavelength | λd |  | 565 |  | nm | IF=20 mA |
| Peak Emission Wavelength | λp |  | 560 |  | nm | IF=20 mA |
| Spectral Line Half Width | Δλ |  | 25 |  | nm | IF=20 mA |
| Viewing Angle | 2θ1/2 |  | 50 |  | deg | IF=20 mA |

Note :

- The luminous intensity data and λP  is survey values with the machine JF-Ⅱ, JS-2000.

- 2θ1/2 is the clip angle at which the luminous intensity half the axial luminous intensity.

Package Dimensions : 10 mm Round resin mold type

Note :

- All dimensions are in millimeters(mm)

- Tolerance is ±0.25mm unless otherwise noted

Typical Characteristic Curves :