DATA SHEET for LED

LUB81043

RoHS

| Part No. | LUB81043 |  |  |
| --- | --- | --- | --- |
| ###### Emitted Color |  | ###### Lens Color | ###### Chip Material |
| Ultra Super Blue |  | Water Clear | InGaN |

Package Dimensions :

Note :

- All dimensions are in millimeters(mm)

- Tolerance is ±0.25mm unless otherwise noted

Absolute Maximum Rating (Ta = 25℃)

| Parameter | Symbol | Maximum Rating | Unit |
| --- | --- | --- | --- |
| Power Dissipation | Pd | 80 | mW |
| Pulse Forward Current (Duty 1/10 @ 1kHz) | IFP | 70 | mA |
| Continuous Forward Current | IF | 20 | mA |
| Reverse Voltage | VR | 5 | V |
| Operation Temperature | Topr | －30℃∼85℃ | ℃ |
| Storage Temperature | Tstg | －40℃∼100℃ | ℃ |
| Soldering Temperature | Tsol | 260±5 | ℃ |

Note : Soldering Time ≤5 seconds

Electron-Optical Characteristics (Ta = 25℃)

| Parameter | Symbol | Min. | Typ. | Max. | Unit | Test Condition |
| --- | --- | --- | --- | --- | --- | --- |
| Luminous Intensity | IV |  | 4,000 |  | mcd | IF=20 mA |
| Forward Voltage | VF |  | 3.2 | 3.8 | V | IF=20 mA |
| Reverse Current | IR |  |  | 50 | μA | VR=5V |
| Dominant Wavelength | λd |  | 472 |  | nm | IF=20 mA |
| Peak Emission Wavelength | λp |  | 467 |  | nm | IF=20 mA |
| Spectral Line Half Width | Δλ |  | 30 |  | nm | IF=20 mA |
| Viewing Angle | 2θ1/2 |  | 25 |  | deg | IF=20 mA |

Typical Electro- Optical Characteristic Curves :

Reliability Test Items and Conditions

| # Test Item | Test Conditions | Duration | Sample | Ac/Re |
| --- | --- | --- | --- | --- |
| Temperature Cycle | -40℃(30 min.) ~ 25℃(5 min.) ~ 100℃(30 min.) ~ 25℃(5 min.) | 50 clycles | 100 | 0/1 |
| High Temp. Storage | Ta=100℃ | 1,000 hours | 100 | 0/1 |
| Temp. & Humidity Test | Ta=85℃ / RH=85% | 1,000 hours | 100 | 0/1 |
| Low Temp. Storage | Ta=-40℃ | 1,000 hours | 100 | 0/1 |
| Operating Life Test | Ta=25±5℃ / DC IF=20mA | 1,000 hours | 100 | 0/1 |
| Solder Heat | Tsol=260±5℃，10s | 1 time | 20 | 0/1 |

Precautions in Use :

A. Soldering Conditions

1. Maximum allowable soldering conditions are.

Solder dipping : 260 ℃ max., 5 seconds max., one time.

Soldering iron : 350 ℃ max., 5 seconds max., one time.

2. In soldering, do not put any stress on the lead frame, particularly when heated.

B. Lead frame Forming and Use

1. When mounting the LEDs onto a printed circuit board ,the holes on the circuit board should be exactly aligned with the leads of the LEDs.

2. Please avoid conditions which may cause the LED to corrode, tarnish or discolor. This corrosion or discoloration may cause difficulty during soldering operations. It is recommended that the LEDs be used as soon as possible.

3. Please avoid rapid transitions in ambient temperature, especially, in high humidity environments.

C. Static Electricity

1. These products are sensitive to static electricity charge, and users are required to handle with care. Particularly, if an current and or voltage which exceeds the Absolute Maximum Rating of Products is applied, the overflow in energy may cause damage to, or possibly result in electrical destruction of, the Products. The customer is requested to take adequate countermeasures against static electricity charge and surge when handling Products.

2. Proper grounding of Products , use of conductive mat, conductive working uniform and shoes, and conductive containers　are effective against static electricity and surge.

3. Ground low-resistance areas where the product contacts, such as metal surfaces of the work platform, with a conductive mat (surface resistance 106-108 Ω ).

4. A tip of soldering iron is requested to be grounded. An ionizer should also be installed where risk of static generation is high.

Notes :

1. Above specification may be changed without notice. We will reserve authority on material change for above specification.

2. When using this product, please observe the absolute maximum ratings and the instructions for the specification sheets. We assume no responsibility for any damage resulting from use of the product which does not comply with the instructions included in the specification sheets.